高性能伺服式数字步进电机驱动器

FOR 32-bit DSP

2HD8080

用户手册

注意

！阅读

请详细阅读本说明书后，再进行安装连接。

●安全注意事项●

本驱动器为直流电源供电，请确认电源正负极后上电

请勿带电插拔电源以及电机线缆

请勿在驱动器内部混入镙丝、金属屑等导电性异物或可燃性异物

驱动器为功率电器，请保持工作环境的散热通风

目录

- 性能简介…………………………………………………………………第1 页

- 型号说明…………………………………………………………………第2 页

- 特殊功能列表……………………………………………………………第2 页

四、控制信号以功率信号接口说明…………………………………………第2 页

五、拨码开关设置……………………………………………………………第4 页

六、状态指示灯..……………………………………………………………第5 页

七、外形尺寸..………………………………………………………………第5 页

八、驱动器使用注意事项……………………………………………………第6 页

一 性能简介

高性能伺服式数字步进电机驱动器 2HD8080 采用德州仪器32bit数字处理器，引入控制理论中实时电流跟踪控制，实现电流闭环。8路独立的同相PWM无差拍输出，独立控制全桥的上下臂，电流控制更精确。同时加入200K的基频，彻底消除次谐波，并加入电子阻尼技术以及实时矢量合成后的PWM输出。使得电机的低频、倍频共振、高频自激振动、电磁噪音较传统开环驱动器有很大的改善。

2HD8080 步进电机驱动器有以下几个显著特点

1：实时矢量合成的8路独立的同相PWM无差拍输出，使得电机运行更平稳，噪声更小。

2：电流闭环控制，可实现缺相保护，避免电机在缺相情况下继续振动运行。

3：可扩展的位置闭环控制，预留编码器接口，可实现误差补偿、堵转保护。

4：独特的断电相位记忆功能，避免开机时电机寻找相位抖动。

5：支持交直流电源输入DC 20-80V最大峰值电流可达8 A。

6：全方位的保护功能，使电机运行更安全可靠。

7：超高性能，超低价格，性价比卓越。

二 型号说明

- 两相步进电机

HD  高性能数字驱动器

80  供电电压24V-80V

80  最高相电流8.0A

三  特殊功能列表

| 功能 | 描述 |
| --- | --- |
| 相位记忆 相位清零 | 在脉冲停止时记忆电机相位，驱动器重新上电时电机寻找该相位，避免上电时电机抖动。同时相位可进行清零操作 |
| 报警输出 | 电机运行错误时除了指示灯信号并伴有IO信号报警输出 |
| 缺相保护 | 当电机缺相时，驱动器停止电机运行，防止电机缺相时抖动运行 |
| 内部脉冲 | 通过拨码开关可选择内部10K（可设置）脉冲输入 |

四 控制信号以功率信号接口说明

功率信号接口

| GND | 直流电源地 |
| --- | --- |
| VCC | 直流电源正极 |
| A+ A- | 电机A相 |
| B+ B- | 电机B相 |

电机信号接口

控制信号接口

五 拨码开关设置

| 电流选择 | SW6 | SW7 | SW8 |
| --- | --- | --- | --- |
| 1．8A | ON | ON | ON |
| 2．4A | ON | ON | OFF |
| 3．0A | ON | OFF | ON |
| 3．6A | ON | OFF | OFF |
| 4．2A | OFF | ON | ON |
| 4．8A | OFF | ON | OFF |
| 5．4A | OFF | ### OFF | ON |
| 6．0A | OFF | OFF | OFF |

| 细分选择 | SW2 | SW3 | # SW4 | ## SW5 |
| --- | --- | --- | --- | --- |
| 2 | ON | ON | ON | ON |
| 4 | ON | ON | ON | ### OFF |
| 8 | ON | ON | OFF | ON |
| 16 | ON | ON | OFF | OFF |
| 32 | ON | OFF | ON | ON |
| 64 | ON | OFF | ON | OFF |
| 128 | ON | OFF | OFF | ON |
| 256 | ON | OFF | OFF | OFF |
| 5 | OFF | ON | ON | ON |
| 10 | OFF | ON | ON | OFF |
| 20 | OFF | ON | OFF | ON |
| 25 | OFF | ON | OFF | OFF |
| 50 | OFF | OFF | ON | ON |
| 100 | OFF | OFF | ON | OFF |
| 200 | OFF | OFF | OFF | ON |
| A-B | OFF | OFF | OFF | OFF |

SW1=ON 内部10K脉冲输入 SW1=OFF 外部脉冲输入

六　状态指示灯

状态指示 LED1: 亮-工作正常               暗-工作异常

LED2: 亮-有脉冲输入             暗-无脉冲输入

报警指示 LEDA: 绿-电机A相缺相            红-电机B相缺相

LEDB: 绿-过压>80V  或 欠压<20V  红-驱动器过流

七　外形尺寸